# 2022 Dodge Charger / Charger SRT

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### General Information

Vehicle Type	E-segment sedan, rear-wheel drive (RWD), all-wheel drive (AWD)

Assembly Plant	Brampton Assembly Plant, Ontario, Canada

EPA Vehicle Class	Large car

Introduction Date	2014 as a 2015 model

### Body and CHASSIS

Layout	Longitudinal front engine, RWD or AWD

Construction	Unitized steel body and aluminum hood

### Engine: 3.6-Liter PENTAStar V-6

Availability	Standard — SXT, SXT AWD, GT and GT AWD

Type and Description	60-degree V-type, liquid-cooled

Displacement	220 cu. in. (3,604 cu. cm)

Bore x Stroke	3.78 x 3.27 in. (96.0 x 83.0 mm)

Valve System	Chain-driven DOHC, 24 valves, hydraulic end-pivot roller rockers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Aluminum deep-skirt block with aluminum-alloy heads

Compression Ratio	10.2:1

Power (SAE net)	292 bhp (218 kW) @ 6,350 rpm

300 bhp (224 kW) @ 6,350 rpm — SXT AWD, GT and GT AWD

Torque (SAE net)	260 lb.-ft. (353 N•m) @ 4,800 rpm

264 lb.-ft. (358 N•m) @ 4,800 rpm — SXT AWD, GT and GT AWD

Max. Engine Speed	6,400 rpm (electronically limited)

Fuel Requirement	Unleaded regular, 87 octane (R+M)/2

Oil Capacity	6 quarts (5.7 liters)

Coolant Capacity	14.0 quarts (13.25 liters)

Emission Controls 	Dual three-way catalytic converters, heated oxygen sensors 
and internal engine features

### Engine: 3.6-Liter PENTAStar V-6 (CONTINUED)

Max. Gross Trailer Weight	1,000 lbs. (454 kg)

EPA Fuel Economy mpg (city/hwy/combined)	19/30/23 (RWD); 18/27/21 (AWD)

Engine Assembly Plant	Trenton South Engine Plant, Trenton, Michigan

### ENGINE: 5.7-Liter HEMI® V-8 WITH FUEL SAVER TECHNOLOGY

Availability	Standard — R/T models

Type and Description	90-degree V-type, liquid-cooled with variable-cam timing (VCT)

Displacement	345 cu. in. (5,654 cu. cm)

Bore x Stroke	3.92 x 3.58 (99.5 x 90.9)

Valve System	Pushrod-operated overhead valves, 16 valves, eight deactivating 
and eight conventional hydraulic lifters, all with roller followers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, 
aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	10.5:1

Power (SAE net)	370 bhp (276 kW) @ 5,250 rpm

Torque (SAE net)	395 lb.-ft. (536 N•m) @ 4,200 rpm

Max. Engine Speed	5,800 rpm (electronically limited)

Fuel Requirement	Unleaded midgrade, 89 octane (R+M)/2 — recommended

Unleaded regular, 87 octane (R+M)/2 — acceptable

Oil Capacity	7 quarts (6.6 liters)

Coolant Capacity	14.7 quarts (13.9 liters)

Emission Controls 	Dual close-coupled three-way catalytic converters, 
quad heated oxygen sensors and internal engine features

Exhaust Type 	Dual 2.75-in. straight-thru exhaust system using twin electronic 
exhaust valves (EEV); features 4-in. chrome round tips

Max. Gross Trailer Weight	1,000 lbs. (454 kg)

EPA Fuel Economy mpg (city/hwy/combined)	16/25/19

Engine Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

### ENGINE: 392 HEMI V-8 POWERED BY SRT

Availability	Standard — Scat Pack models

Type and Description	90-degree V-type, liquid-cooled

Displacement	392 cu. in. (6,417 cu. cm)

Bore x Stroke	4.09 x 3.72 (103.9 x 94.5)

Valve System	Pushrod-operated overhead valves, 16 valves with sodium-filled exhaust valves and hollow stem intake valves, 16 conventional hydraulic lifters, all with roller tips

Fuel Injection	Sequential, multiport, electronic, returnless; automatic transmission features Fuel Saver mode

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, 
aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	10.9:1

Power (SAE net)	485 bhp (362 kW) @ 6,100 rpm

Torque (SAE net)	475 lb.-ft. (644 N•m) @ 4,100 rpm

Max. Engine Speed	6,400 rpm (electronically limited)

Fuel Requirement	Premium 91 octane (R+M)/2 — required

Oil Capacity	7 quarts (6.6 liters)

Factory Oil Fill	0W-40 Pennzoil Ultra synthetic

Coolant Capacity	15 quarts (14.4 liters)

Emission Controls 	Dual close-coupled three-way catalytic converters, quad-heated oxygen sensors and internal engine features

Exhaust Type	Dual, 2.75 in. electronically controlled, performance-tuned 
Active Exhaust system with 4-in. chrome round tips

Max. Gross Trailer Weight	Not rated for trailer-tow

EPA Fuel Economy mpg (city/hwy/combined)	15/24/18

### ENGINE: SUPERCHARGED 6.2-LITER HEMI HELLCAT V-8

Availability	Standard — SRT Hellcat

Type and Description	90-degree V-type, liquid-cooled

Displacement	376 cu. in. (6,166 cu. cm)

Bore x Stroke	4.09 x 3.58 (103.9 x 90.9)

Valve System	Pushrod-operated overhead valves, 16 valves with sodium-filled exhaust valves and hollow stem intake valves, 16 conventional hydraulic lifters, all with roller tips

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, 
unique aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	9.5:1

### ENGINE: SUPERCHARGED 6.2-LITER HEMI HELLCAT V-8 (CONTINUED)

Power 	717 hp (535 kW) @ 6,100 rpm

Torque 	650 lb.-ft. (881 N•m) @ 4,800 rpm

Max. Engine Speed	6,200 rpm (electronically limited)

Fuel Requirement	Premium 91 octane (R+M)/2 — required

Oil Capacity	7 quarts (6.6 liters)

Factory Oil Fill 	0W-40 Pennzoil Ultra synthetic

Coolant Capacity	15 quarts (14.4 liters)

Induction System	Combined airflow rate of 1,094 CFM from mail-slot grille opening

Intercooler System	Separate low-temp cooling system with dual water-to-air intercoolers and a high-flow variable-speed electric water pump

When engine is shut down, radio-selectable Race Cooldown feature turns on radiator fan and intercooler water pump to reduce intercooler coolant temperature

Emission Controls 	Dual close-coupled three-way catalytic converters heated wide range upstream and switching downstream O2 sensors and internal engine features

Exhaust Type 	Dual 2.75-in. straight-thru exhaust system using twin electronic 
exhaust valves (EEV) with 4-in. chrome round tips

Max. Gross Trailer Weight	Not rated for trailer-tow

EPA Fuel Economy mpg (city/hwy/combined)	12/21/15

Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	9.5:1

### ENGINE: SUPERCHARGED 6.2-LITER HEMI HELLCAT HIGH OUTPUT V-8

Availability	Standard — Charger SRT Hellcat Redeye

Type and Description	90-degree V-type, liquid-cooled

Displacement	376 cu. in. (6,166 cu. cm.)

Bore x Stroke	4.09 x 3.58 (103.9 x 90.9)

Valve System	Pushrod-operated overhead valve, 16 valves with sodium-filled exhausts and hollow stem intakes, 16 hydraulic roller lifters

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	9.5:1

Power 	797 bhp (595 kW) @ 6,300 rpm

Torque 	707 lb.-ft (959 N•m) @ 4,500 rpm

### ENGINE: SUPERCHARGED 6.2-LITER HEMI HELLCAT HIGH OUTPUT V-8 (CONTINUED)

Max. Engine Speed	6,500 rpm

Fuel Requirement	Premium 91 octane (R+M)/2 — required

Oil Capacity	7 quarts (6.6 liters)

Factory Oil Fill	0W-40 Pennzoil Ultra synthetic

Coolant Capacity	15 quarts (14.4 liters)

Induction System 	Combined airflow rate of 1,130 cubic feet per minute (CFM) from:

Performance hood with functional air induction

Mail-slot grille opening

Air box opening near wheel liner

Intercooler System	Separate low-temp cooling system with dual water-to-air intercoolers and a high-flow variable-speed electric water pump

SRT Power Chiller™ liquid to air intercooler chiller system is activated and redirects air conditioning refrigerant to chill the intercooler coolant (via radio button)

When engine is shut down, radio-selectable Race Cooldown feature turns on radiator fan and intercooler water pump to reduce intercooler coolant temperature

Emission Controls 	Dual close-coupled three-way catalytic converters heated wide range upstream and switching downstream O2 sensors

Exhaust Type	Dual 2.75-in. straight-though exhaust system with twin electronic exhaust valves (EEV) with rectangular Black Vapor-chromed tips

Max. Gross Trailer Weight	Not rated for trailer-tow

EPA Fuel Economy mpg (city/hwy/combined)	12/21/15

### TRANSMISSION: TORQUEFLITE 8HP50 EIGHT-SPEED AUTOMATIC

Availability	Standard with 3.6-liter Pentastar V-6 engine, RWD

Description	Adaptive electronic control, Sport Mode and available paddle-shifted driver-interactive manual control. Five-clutch pack design with only two open clutches in any gear. Off-center line pump with low-viscosity oil for reduced spin loss. Torque converter lock with turbine torsional damper for low lock-up speeds in first through eighth gear

### TRANSMISSION: TORQUEFLITE 8HP50 EIGHT-SPEED AUTOMATIC (CONTINUED)

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.10

4th	1.67

5th	1.29

6th 	1.00

7th 	0.84

8th 	0.67

Reverse	3.30

Final Drive Ratio	2.62 — standard

3.08 — optional

Overall Top Gear	1.76 — standard

2.06 — optional

### TRANSMISSION: TORQUEFLITE 850RE EIGHT-SPEED AUTOMATIC

Availability	Standard with 3.6-liter Pentastar V-6 engine, AWD

Description	Adaptive electronic control, Sport Mode or paddle-shifted driver-interactive manual control. Five-clutch pack design with only two open clutches in any gear. Off-center line pump with low-viscosity oil for reduced spin loss. Torque converter lock with turbine torsional damper for low lock-up speeds in first through eighth gear

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.10

4th	1.67

5th	1.29

6th 	1.00

7th 	0.84

8th 	0.67

Reverse	3.30

Final Drive Ratio	3.08

Overall Top Gear	2.06

### TRANSMISSION: TORQUEFLITE 8HP70 EIGHT-SPEED AUTOMATIC

Availability	Standard with 5.7-liter and 392 HEMI V-8 engines

Description	Adaptive electronic control, configurable drive modes and paddle-shifted driver-interactive manual control, five-clutch pack design with only two open clutches in any gear. Off-center line pump with low-viscosity oil for reduced spin loss, torque converter lock with turbine torsional damper for low lock-up speeds in first through eighth gear

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.10

4th	1.67

5th	1.29

6th 	1.00

7th 	0.84

8th 	0.67

Reverse	3.32

Final Drive Ratio	2.62 — standard with 5.7-liter

3.09 — asymmetric limited-slip, performance-tuned, 
standard with 392 HEMI V-8

Overall Top Gear	1.75 — (with 2.62 final drive ratio)

2.07 — (with 3.09 final drive ratio)

### TRANSMISSION: TORQUEFLITE 8HP90 EIGHT-SPEED AUTOMATIC

Availability	Standard on SRT Hellcat, SRT Hellcat Redeye

Description	Adaptive electronic control with full manual control via gear selector or paddle shifters, with three SRT-unique selectable modes: Street, Sport and Track (features performance shifting and gear holding feature)

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.10

4th	1.67

5th	1.29

6th	1.00

7th	0.84

8th	0.67

Reverse	3.32

Final Drive Ratio	2.62 — asymmetric limited-slip, performance-tuned

Overall Top Gear	1.76

### TRANSFER CASE

Availability	Standard — SXT AWD and GT AWD

Type	Active, fully variable with front-axle disconnect

Center Differential	Planetary

Torque Split, Front/Rear	Fully variable

### Electrical System

Alternator	160-amp — standard on SXT

180-amp — standard on SXT AWD, GT, GT AWD and R/T models; 
optional on SXT

220-amp — standard on Scat Pack models, SRT Hellcat and 
SRT Hellcat Redeye

Battery	H7 Case, 730 CCA, maintenance-free

### Steering

Type 	Electric power steering (EPS) with multi-mode assist

Overall Ratio	16.5:1 — SXT AWD

14.4:1 — all RWD models

Turn Circle (curb-to-curb)	37.7 ft. — SXT, GT, R/T models

38.7 ft. — SXT AWD and GT AWD

37.5 ft. — Scat Pack

39.0 ft. — Scat Pack Widebody, SRT Hellcat and SRT Hellcat Redeye

Steering Turns (lock-to-lock)	2.6 — SXT, GT, R/T models

3.1 — SXT AWD and GT AWD

2.6 — Scat Pack models

2.5 — SRT Hellcat and SRT Hellcat Redeye

### Suspension

Front 	Independent short- and long-arm (SLA) with high upper “A” arm

Lateral and diagonal lower links (RWD models), one-piece lower-control arms (AWD models)

Coil spring over gas-charged monotube shock absorbers

Bilstein Adaptive Damping Suspension (ADS) shock absorbers (available on Scat Pack, standard Scat Pack Widebody and SRT Hellcat models)

32 mm (34 mm for SRT models) hollow stabilizer bar

### Suspension (CONTINED)

Rear	Five-link independent with coil springs

Link-type 19 mm (22 mm on Widebody models) hollow stabilizer bar

Gas-charged monotube shock absorbers

Gas-charged Bilstein ADS shock absorbers (available on Scat Pack, 
standard Scat Pack Widebody and SRT Hellcat models)

Isolated suspension cradle

Touring-tuned Suspension 	Standard — SXT

Normal-duty Suspension 	Standard — SXT AWD and GT AWD

Sport-tuned Suspension 	Optional — SXT

Performance Suspension	Standard — GT and R/T models

High-performance Suspension
(included with Performance Handling)	Standard — Scat Pack models; optional on GT and R/T models

Widebody Competition Suspension 
(included with Widebody Package)	Standard — SRT Hellcat and SRT Hellcat Redeye; available on Scat Pack

SRT-tuned Three-mode ADS	Standard — SRT Hellcat and SRT Hellcat Redeye; available on Scat Pack

Auto Mode: sporty, compliant ride

Sport Mode: firm, improved handling

Track Mode: firmest, maximum handling

### Dimensions & Capacities

Wheelbase	120 (3,048)

Track, Front	63.4 (1,611) — SXT, GT and R/T

63.3 (1,607) — SXT AWD and GT AWD

64.0 (1,625) — Scat Pack

65.6 (1,667) — Scat Pack Widebody, SRT Hellcat and SRT Hellcat Redeye

Track, Rear	63.8 (1,620) — SXT, GT and R/T

63.4 (1,611) — SXT AWD and GT AWD

63.7 (1,617) — Scat Pack

65.3 (1,658) — Scat Pack Widebody, SRT Hellcat and SRT Hellcat Redeye

Overall Length	198.4 (5,040) — SXT and SXT AWD

200.8 (5,100) — GT, GT AWD, R/T and Scat Pack

201.0 (5,105) — Scat Pack Widebody, SRT Hellcat and SRT Hellcat Redeye

Overall Width (with mirrors)	82.7 (2,100)

Overall Width (without mirrors)	75.0 (1,905) — SXT, SXT AWD, GT, GT AWD, R/T and Scat Pack

78.3 (1,989) — Scat Pack Widebody, SRT Hellcat and SRT Hellcat Redeye (at widest point of vehicle, rear flares)

### Dimensions & Capacities (CONTINUED)

Overall Height	58.5 (1,485) — SXT AWD and GT AWD

57.8 (1,467) — SXT, GT, R/T and Scat Pack

57.6 (1,462) — SRT Hellcat and SRT Hellcat Redeye

Frontal Area, sq. ft. (sq. m)	25.8 (2.4) — SXT

26.0 (2.4) — SXT AWD

26.2 (2.4) — GT, GT AWD, R/T and Scat Pack

27.2 (2.5) — Scat Pack Widebody, SRT Hellcat and SRT Hellcat Redeye

Ground Clearance	5.2 (130.9)

4.5 (115.2) — SRT Hellcat and SRT Hellcat Redeye

Drag Coefficient (Cd)	0.304 Cd — SXT

0.330 Cd — SXT AWD

0.335 Cd — GT, GT AWD, R/T and Scat Pack

0.380 Cd — Scat Pack Widebody

0.376 Cd — SRT Hellcat and SRT Hellcat Redeye

Curb Weight, lb. (kg)	3,940 (1,787) — SXT

4,210 (1,909) — SXT AWD

4,039 (1,832) — GT

4,247 (1,926) — GT AWD

4,273 (1,938) — R/T models

4,361 (1,978) — Scat Pack models

4,594 (2,083) — SRT Hellcat

4,591 (2,082) — SRT Hellcat Redeye

Weight Distribution, percent F/R	52/48 — SXT and GT

53/47 — SXT AWD and GT AWD

55/45 — R/T models

55/45 — Scat Pack models

57/43 — SRT Hellcat

57/43 — SRT Hellcat Redeye

Fuel Tank Capacity, gallons (liters)	18.5 (70.0)

### Accommodations

Seating Capacity, F/R	2/3 — standard

Front

Headroom	38.6 (981)

36.9 (938) with sunroof

Legroom	41.8 (1,061)

Shoulder room	59.5 (1,510)

Hip room	56.2 (1,428)

Seat travel	Driver — 10.0 (255)

Passenger — 8.66 (220)

Recliner angle range, deg.	Power — 70.7

Manual — 70.0

EPA front volume index
cu. ft. (cu. m)	55.6 (1.57)

Rear Seat

Headroom	36.6 (930)

Legroom	40.1 (1,019)

Knee clearance	3.9 (99)

Shoulder room	57.9 (1,472)

Hip room	56.1 (1,425)

EPA second row interior volume, cu. ft. (cu. m)	49.2 (1.39)

Total interior volume, cu. ft. (cu. m)	104.7 (2.97)

EPA Luggage Compartment Volume, 
cu. ft. (cu. m)	16.5 (0.47)

EPA Interior Volume Index, cu. ft. (cu. m)	120.8 (3.42)

Trunk Liftover Height		30.1 (764)

### Tires

Availability 	Standard on SXT

Size and type	P215/65R17 BSW Low-rolling Resistance All-season Touring

Mfr. and model	Michelin Energy Saver

Revs per mile (km)	745 (463)

Availability	Standard on SXT AWD

Size and type	235/55R19 BSW All-season Performance

Mfr. and model	Michelin Primacy MXM4

Revs per mile (km)	715 (444)

### Tires (CONTINUED)

Availability	Standard on GT, GT AWD, R/T and Scat Pack; optional on SXT 
and SXT AWD (packaged with Blacktop Package, Plus Group)

Size and type	245/45ZR20 BSW All-season Performance

Mfr. and model	Firestone Firehawk GTV

Revs per mile (km)	726 (449)

Availability	Optional on Scat Pack (packaged with Daytona Edition Group, 
Dynamics Package)

Size and type	275/40ZR20 All-season Performance

Mfr. and model	Pirelli P Zero All-season

Revs per mile (km)	718 (446)

Availability	Optional on Scat Pack

Size and type	275/40ZR20 Summer

Mfr. and model	Pirelli P Zero

Revs per mile, (km)	718 (446)

Availability	Standard on Scat Pack (with Widebody Package), 
SRT Hellcat and SRT Hellcat Redeye

Size and type	305/35ZR20 All-season Performance

Mfr. and model	Pirelli P Zero All-season

Revs per mile, (km)	731 (454)

Availability	Optional on Scat Pack (with Widebody Package), 
SRT Hellcat and SRT Hellcat Redeye

Size and type	305/35ZR20 Three-season Performance

Mfr. and model	Pirelli P Zero

Revs per mile, (km)	731 (454)

### WHEELS

Availability	Standard on SXT

Type and material	Fine Silver / Aluminum

Size	17 x 7 in.

Availability	Standard on SXT AWD

Type and material	Satin Carbon / Aluminum

Size	19 x 7.5 in.

Availability	Optional on SXT AWD (included with Blacktop Package)

Type and material	Black Noise / Aluminum

Size	19 x 7.5 in.

### WHEELS (CONTINUED)

Availability	Standard on GT; optional on SXT (included with Plus Group)

Type and material	Satin Carbon, wide spoke design / Aluminum

Size	20 x 8 in.

Availability	Standard on GT AWD; optional on SXT AWD (included with Plus Group)

Type and material	Satin Carbon / Aluminum

Size	20 x 8 in.

Availability	Standard on R/T; optional on GT (included with Plus Group)

Type and material	Satin Carbon, tight spoke design / Aluminum

Size	20 x 8 in.

Availability	Optional on SXT, GT and R/T (included with Blacktop Package)

Type and material	Black Noise / Aluminum

Size	20 x 8 in.

Availability	Optional on SXT AWD and GT AWD (included with Blacktop Package)

Type and material	Black Noise / Aluminum

Size	20 x 8 in.

Availability	Optional on GT and R/T (included with Performance Handling Group)

Type and material	Lights Out / Aluminum

Size	20 x 9 in.

Availability	Optional on GT and R/T (included with Performance Handling Group, 
Daytona Edition Group)

Type and material	Low Gloss Black / Aluminum

Size	20 x 9 in.

Availability	Optional on R/T (included with Plus Group)

Type and material	Machined with Granite pockets / Aluminum

Size	20 x 8 in.

Availability	Standard on Scat Pack

Type and material	Forged Low Gloss Granite / Aluminum

Size	20 x 9 in.

Availability	Optional on Scat Pack (included with Daytona Edition Group, 
Dynamics Package)

Type and material	Black Noise / Aluminum

Size	20 x 9.5 in.

Availability 	Standard on Scat Pack (with Widebody Package)

Type and material	Devil’s Rim / Aluminum

Size	20 x 11 in.

### WHEELS (CONTINUED)

Availability 	Standard on SRT Hellcat; optional on Scat Pack (with Widebody Package) and SRT Hellcat Redeye

Type and material	Carbon Black / Aluminum

Size	20 x 11 in.

Availability 	Standard on SRT Hellcat Redeye; optional on SRT Hellcat

Type and material	Lightweight Carbon Black / Aluminum

Size	20 x 11 in.

Availability 	Optional on SRT Hellcat and SRT Hellcat Redeye

Type and material	“Warp Speed” Granite / Aluminum

Size	20 x 11 in.

Availability 	Optional on SRT Hellcat and SRT Hellcat Redeye

Type and material	Brass Monkey / Aluminum

Size	20 x 11 in.

### Brakes

Sport Brake Package 	Standard on SXT

Front

Rotor size and type	12.6 x 1.1 (320 x 28) vented

Caliper size and type	2.36 (60) single piston sliding with aluminum housing

Swept area	210 sq. in. (1,358 sq. cm)

Rear

Rotor size and type	12.6 x 0.4 (320 x 10) solid

Caliper size and type	1.65 (42) single piston sliding with aluminum housing

Swept area	264 sq. in. (1,703 sq. cm)

Performance Brake Package 	Standard on SXT AWD, GT, GT AWD and R/T

Front

Rotor size and type	13.6 x 1.26 (345 x 28) vented

Caliper size and type	1.65 (42) dual piston sliding with aluminum housing

Swept area	249 sq. in. (1,606 sq. cm)

Rear

Rotor size and type	12.6 x 0.87 (320 x 22) vented

Caliper size and type	1.65 (42) single piston sliding with aluminum housing

Swept area	264 sq. in. (1,703 sq. cm)

### Brakes (CONTINUED)

Brembo High-performance Brake Package 	Standard on Scat Pack; optional on GT, R/T models

Front

Rotor size and type	14.2 x 1.26 (360 x 32.0) vented

Caliper size and type	1.73 (44) Brembo four-piston fixed with all aluminum housing

Swept area	351 sq. in. (2,261 sq. cm)

Rear

Rotor size and type	13.8 x 1.10 (350 x 28) vented

Caliper size and type	1.26 (32) Brembo four-piston fixed with aluminum housing

Swept area	316 sq. in. (2,036 sq. cm)

SRT/Brembo Ultra-high-performance 
Brake Package	Standard on SRT Hellcat and SRT Hellcat Redeye

Included on Scat Pack with Daytona Edition Group and Dynamics Package

Front

Rotor size and type	15.7 x 1.34 (400 x 34.0) two-piece aluminum hat, 
directional vane vented and slotted

Caliper size and type	1.42 (36 mm) Brembo six-piston fixed monoblock aluminum

Swept area	442 sq.in. (2,852 sq. cm.)

Rear

Rotor size and type	13.8 x 1.10 (350 x 28) vented

Caliper size and type	1.26 (32) Brembo four-piston fixed with aluminum housing

Swept area	316 sq. in. (2,036 sq. cm.)

Four-wheel Antilock Brake System (ABS) 
and Traction Control	Standard

Electronic Stability Control (ESC) 
and Brake Assist	Standard

Brake Knockback Mitigation	Standard

Hill-start Assist (HSA)	Standard

Three-mode (Normal, Sport, Off) 
ESC/Traction Control Mode Selection	Standard on SXT AWD, GT, GT AWD and R/T models

Four-mode (Street, Sport, Track, Off) 
ESC/Traction Control Mode Selection	Standard on Scat Pack, SRT Hellcat and SRT Hellcat Redeye

Power-assist Type	8 + 9 (203 + 209) tandem-diaphragm vacuum booster

###