# 2022 Jeep® Compass

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Body Style	Sport-utility Vehicle (SUV)

Assembly Plant	Toluca, Mexico

EPA Vehicle Class	Multipurpose vehicle

### BODY AND CHASSIS

Layout	Transverse front engine, 4x2 and 4x4

Construction	Steel uniframe

### ENGINE: 2.4-LITER TIGERSHARK WITH MULTIAIR2 WITH ENGINE STOP-START (ESS)

Availability	Standard — all models

Type and Description	In-line four-cylinder, 16-valve MultiAir with multiport fuel injection

Displacement	144 cu. in. (2,360 cc)

Bore x Stroke	3.46 x 3.82 (88 x 97)

Valve System	SOHC, four valves per cylinder

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Aluminum block, aluminum cylinder head

Compression Ratio	10:1

Power (SAE net)	177 hp @ 6,400 rpm

Torque (SAE net)	172 lb.-ft. @ 3,900 rpm

Max. Engine Speed	6,400 rpm

Fuel Requirement	Unleaded regular, 87 octane

Oil Capacity	5.5 quarts (5.2 liters) (total)

Coolant Capacity	6.8 quarts (6.45 liters)

Emission Controls	Single catalytic converter, heated wide band lambda sensor upstream and mid-catalyst heated oxygen sensor

Fuel Economy (city/hwy/combined)	22 / 31 / 25 (FWD, ATX)

22 / 30 / 25 (AWD, ATX)

Towing	2,000 lbs. (907 kg) (not recommended for FWD)

Engine Assembly Plant	Dundee, Mich.

### TRANSMISSION: 948TE NINE-SPEED AUTOMATIC

Availability	2.4-liter Tigershark engine with MultiAir2 (4x4)

Description	Planetary gear train, transverse layout

Ratio Spread	9.81

Gear Ratios

1st	4.71

2nd	2.84

3rd	1.91

4th	1.38

5th	1.00

6th	0.81

7th	0.70

8th	0.58

9th	0.48

Reverse	3.81

Final Drive Ratio	3.734

### TRANSMISSION: AT6 AISIN SIX-SPEED AUTOMATIC

Availability	2.4-liter Tigershark engine with MultiAir2 (4x2)

Description	Planetary gear train, transverse layout

Ratio Spread	6.635

Gear Ratios

1st	4.459

2nd	2.508

3rd	1.556

4th	1.142

5th	0.852

6th	0.672

Final Drive Ratio	3.502

### 4X4 SYSTEMS: JEEP ACTIVE DRIVE, JEEP ACTIVE DRIVE LOW

Availability 	Jeep Active Drive is optional on all models

Jeep Active Drive Low is standard on Trailhawk

Type	Fully disconnecting 4x2 mode with automatic 4x4 engagement

Full-time 4x4 mode with active on demand clutch

### 4X4 SYSTEMS: JEEP ACTIVE DRIVE, JEEP ACTIVE DRIVE LOW (continued)

Operating Modes	Auto 4x2/4x4, neutral

Center Differential Type	None

Terrain Response	Unique tuning in all terrain modes

Crawl Ratio	20:1 (with Jeep Active Drive Low)

### ELECTRICAL SYSTEM

Alternator	160-amp (2.4-liter Tigershark with MultiAir2)

Battery	Dual batteries when equipped with electronic stop-start system: 500-amp (primary), 150-amp (secondary) (2.4-liter Tigershark with MultiAir2 with engine stop-start)

### SUSPENSION

Front 	MacPherson strut, coil springs, flat front steel crossmember, high-strength steel double shell lower control stabilizer bar

Rear	Chapman strut, high-strength steel links, isolated steel rear cradle for 4x4 and not isolated for 4x2, coil springs, stabilizer bar

### STEERING

Type 	Electric power rack and pinion

Overall Ratio	15.7

Turning Diameter (curb-to-curb)	36.3 (11.07) 4x2 and 4x4

35.3 (10.76) Trailhawk 4x4

Steering Turns (lock-to-lock)	2.68 (4x2 and 4x4)

2.76 (Trailhawk 4x4)

### BRAKES

Front

Size and type	12 x 1.1 (305 x 28) vented rotor with 2.36 (60) single-piston floating caliper

Swept area (per caliper)	138.77 sq. in. (895.3 sq. cm), 1 x 60

115.88 sq. in. (747.6 sq. cm), 1 x 57

Rear

Size and type	10.95 x 0.47 (278 x 12) solid rotor with 1.5 (38) single-piston floating caliper

Swept area (per caliper)	101.7 sq. in. (656.4 sq. cm), 1 x 38

Power-assist type	10 (254) vacuum assist

### BRAKES (continued)

Four-wheel antilock brake system (ABS)	Standard

Electronic stability control (ESC)	Standard

Parking brake type	Electric motor

### DIMENSIONS AND CAPACITIES

Wheelbase	103.8 (2,636)

Track, Front	60.7 (1,542)

Track, Rear	60.3 (1,532)

Overall Length	173.4 (4,404)

Overall Width (with folded side mirrors)	73.8 (1,874)

Overall Width (with open side mirrors)	80 (2,033)

Overall Height	64.6 (1,641) without rails; 64.8 (1,647) with rails

Load-floor Height	29.7 (754) (height measured from curb ground to load floor)

Sill-step Height	19.4 (494) (height measured from curb ground to sill trim)

Ground Clearances	4x2 7.7 (196)

4x4 8.1 (206)

Trailhawk 4x4 8.6 (219)

Approach Angle (with air dam) (degrees)	4x2 15.0

4x4 16.1

Trailhawk 4x4 30.4

Breakover Angle (degrees)	4x2 21.6

4x4 22.6

Trailhawk 4x4 23.6

Departure Angle (degrees)	4x2 30.7

4x4 31.4

Trailhawk 4x4 34.0

Curb Weight, lbs. (kg)	3,184 (1,444) 4x2 and 2.4-liter Tigershark with MultiAir2

3,327 (1,509) 4x4 and 2.4-liter Tigershark with MultiAir2

3,633 (1,648) Trailhawk 4x4 with 2.4-liter Tigershark with MultiAir2

Fuel Tank Capacity	13.5 gal. (51 liters)

### ACCOMMODATIONS

Seating Capacity — Front/Rear	2/3

Front Row

Headroom without sunroof	39.2 (995)

Headroom with sunroof	38.6 (981)

Legroom	41.8 (1,046)

Shoulder room	56.7 (1,439)

Hip room	54.1 (1,375)

Seat travel	8.7 (220) fore/aft travel range

Recliner range (degrees)	Manual/power recliner travel: 69 degrees

Manual fold FWD recliner travel: 107.5 degrees

SAE volume (standard roof), cu. ft. (cu. m)	52.9 (1.49) front passenger volume

Second Row

Headroom	38.5 (978)

Legroom	38.3 (973)

Shoulder room	55.1 (1,400)

Hip room	49.2 (1,250)

SAE volume, cu. ft. (cu. m)	46.7 (1.35) second-row passenger volume

Cargo Access

Liftover height	31.1 (792) (height measured from curb ground to liftgate scuff trim)

Maximum cargo width at liftgate opening	42.1 (1,069)

Minimum cargo width at liftgate opening	36.9 (938)

Maximum cargo height at liftgate opening	27.2 (691)

Distance between wheelhouse interior trim	38.1 (968)

SAE Cargo Volume

Rear seats up, cu. ft. (cu. m)	27.2 (0.8)

Rear seats folded, cu. ft. (cu. m)	59.8 (1.7)

EPA interior volume index, cu. ft. (cu. m)	126.7 (3.6)

### TIRES

Availability	Standard — Sport 4x2, Sport 4x4

Size and type	215/65R16 BSW All-season

Mfr. and model	Continental CrossContact LX Sport

Revs per mile (km)	751 (467)

### TIRES (continued)

Availability	Standard — Latitude 4x2, Latitude 4x4

Size and type	225/60R17 BSW All-season

Mfr. and model		Firestone Destination LE2

Revs per mile (km)	733 (455)

Availability	Standard — Altitude 4x2, Altitude 4x4, Limited

Size and type	225/55R18 BSW All-season

Mfr. and model	Continental Pro Contact

Revs per mile (km)	727 (452)

Availability	Standard — Trailhawk

Size and type	215/65R17 on/off-road

Mfr. and model	Falken WildPeak H/T

Revs per mile (km)	715 (445)

Availability	Standard — Latitude LUX

Size and type	225/55R18 BSW All-season

Mfr. and model	Yokohama Geolandar

Revs per mile (km)	727 (452)

Availability	Standard — High Altitude (optional on Limited)

Size and type	235/45R19 BSW All-season

Mfr. and model	Continental Pro Contact

Revs per mile (km)	727 (452)

###