# 2022 Jeep® Grand Cherokee L

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Vehicle Type	Four-door sport-utility vehicle

Assembly Plant	Detroit Assembly Complex – Mack Plant, Detroit, Michigan

EPA Vehicle Class	Multipurpose vehicle

Introduction Date	January 2021 as a 2021 model

### BODY AND CHASSIS

Layout	Front engine, rear- or four-wheel drive

Construction	Steel uniframe

### ENGINE: 3.6-LITER PENTASTAR V-6

Availability	Standard — Laredo, Limited, Overland and Summit

Type and Description	60-degree V-type, liquid-cooled

Displacement	220 cu. in. (3,604 cu. cm)

Bore x Stroke	3.78 x 3.27 (96 x 83)

Valve System	Chain-driven DOHC, 24 valves and hydraulic end-pivot roller rockers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Aluminum deep-skirt block, aluminum alloy heads

Compression Ratio	11.3:1

Power (SAE J2723)	293 hp @ 6,400 rpm

Torque (SAE J2723)	260 lb.-ft. @ 4,000 rpm

Max. Engine Speed	6,700 rpm (electronically limited)

Fuel Requirement	Unleaded regular, 87 octane (R + M)/2

Oil Capacity	5.0 quarts (4.7 liters)

Coolant Capacity	14.0 quarts (13.25 liters)

Emission Controls 	Dual three-way catalytic converters, heated oxygen sensors and internal engine features

Max. Gross Trailer Weight	6,200 lbs. — 2WD and 4WD

### ENGINE: 3.6-LITER PENTASTAR V-6 (CONTINUED)

EPA Fuel Economy mpg (city/hwy/combined)	19/26/21 — 2WD

18/25/21 — 4WD

Assembly Plant	Saltillo South Engine Plant, Saltillo, Mexico

### ENGINE: 5.7-LITER V-8

Availability	Optional — Overland and Summit (4x4 models only)

Type and Description	90-degree V-type, liquid-cooled

Displacement	345 cu. in. (5,654 cu. cm)

Bore x Stroke	3.92 x 3.58 (99.5 x 90.9)

Valve System	Variable-valve timing (VVT), pushrod-operated overhead valves, 
16 valves, eight deactivating and eight conventional hydraulic lifters, 
all with roller followers

Fuel Injection	Sequential, multi-port, electronic, returnless

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	10.5:1

Power (SAE J2723)	357 hp (268 kW) @ 5,150 rpm

Torque (SAE J2723)	390 lb.-ft. (520 N•m) @ 4,250 rpm

Max. Engine Speed	5,800 rpm (electronically limited)

Fuel Requirement	Unleaded mid-grade, 89 octane (R+M)/2 — recommended

Unleaded regular, 87 octane (R+M)/2 — acceptable

Oil Capacity	7 quarts (6.6 liters)

Coolant Capacity	14.5 quarts (13.72 liters)

Emission Controls 	Dual close-coupled three-way catalytic converters, quad-heated-oxygen sensors and internal engine features(a)

Max. Gross Trailer Weight	7,200 lbs. (4x4)

EPA Fuel Economy mpg (city/hwy/combined)	14/22/17 — 4WD

Assembly Plant	Saltillo North Engine Plant, Saltillo, Mexico

- Meets Calif. LEV II+ evaporative emission requirements in CA, NY, MA, ME, VT and 
Federal Bin 5+ Tier II emissions and Clean Fuel Fleet Certification (CCF).

### TRANSMISSION: TORQUEFLITE 8HP70 AUTOMATIC, EIGHT-SPEED OVERDRIVE

Availability	Standard — 3.6-liter and 5.7-liter models

Description	Driver-interactive manual control and electronically modulated torque 
converter clutch

### TRANSMISSION: TORQUEFLITE 8HP70 AUTOMATIC, EIGHT-SPEED OVERDRIVE (CONTINUED)

Gear Ratios

1st	4.71

2nd 	3.14

3rd	2.11

4th 	1.67

5th 	1.29

6th	1.00

7th	0.84

8th 	0.67

Reverse 	3.30

### TRANSFER CASE: MP 3010

Availability	Optional — 3.6-liter V-6 engine

Type	Single-speed electronically shifted

Operating Mode	Full-time Active 4WD

Low Range Ratio	None

Torque Split, Front/Rear	Variable

### TRANSFER CASE: MP 3022

Availability	Optional — 3.6-liter V-6 and 5.7-liter V-8 engines

Type	Two-speed electronically shifted

Operating Modes	4x4 Low (Lock), Neutral; Full-time Active 4x4

Low Range Ratio	2.72

Torque Split, Front/Rear	Variable

### AXLES

Front

Differential type	Conventional with Front Axle Disconnect

Availability	Standard on 4x4 models

Ring gear diameter	7.7 in. (195mm)

Axle ratios	3.45:1 — 3.6-liter V-6 and 5.7-liter V-8

### AXLES (CONTINUED)

Rear

Differential type	Conventional (optional Electronic Limited Slip on 5.7-liter V-8)

Availability	Standard on all engines

Ring gear diameter 	7.7 in. (195mm) — V-6 4x4

9.1 in. (230mm) — V-6 4x2, 5.7-liter V-8

Axle ratios	3.45:1 — 3.6-liter V-6 and 5.7-liter V-8

### ELECTRICAL SYSTEM

Alternator	160-amp; 180-amp; 220-amp or 240-amp depending on model

Battery	Maintenance-free

H6 650 CCA + Auxiliary 200 CCA on engine stop-start (ESS) — 
Laredo, Limited

H7 700 CCA + Auxiliary 200 CCA on engine stop-start (ESS) — Overland, Summit

H7 700 CCA on V-8 (not equipped with ESS)

### SUSPENSION

Front	Multi-link independent front suspension, coil springs, gas-charged, 
twin-tube coilover shock absorbers, all aluminum arms and knuckles

Rear	Multi-link rear suspension, coil spring, twin tube shocks (including load leveling for towing), all aluminum arms and knuckles (including extruded aluminum spring links)

### STEERING

Type	Electric rack-and-pinion steering

Overall Ratio	16.9:1 (V-6 4x2); 15.6:1 (all other vehicles)

Turning Diameter (curb-to-curb) 	39.6 ft. (12.1 m)

Steering Turns (lock-to-lock)	2.9 (V-6 4x2), 3.0 (all other vehicles)

### BRAKES

Type	Electric boost

Availability	Standard

Front

Rotor size and type 
Inches (mm)	13.94 x 1.10 (354 x 28) vented disc

Caliper size and type 
Inches (mm)	2.00 (51) two-piston pin-slider caliper

Swept area, (sq. in. / sq. cm)		377 (2,432.25)

### BRAKES (CONTINUED)

Rear

Rotor size and type
Inches (mm)	13.78 x 0.87 (350 x 22) vented disc

Caliper size and type
Inches (mm)	2.00 (51) single-piston pin-slider EPB caliper

Swept area, (sq. in. / sq. cm)	293.28 (1,892.12)

### DIMENSIONS AND CAPACITIES

Wheelbase	121.7 (3,091)

Track, Front	65.4 (1,660)

Track, Rear	65.4 (1,660)

Overall Length	204.9 (5,204)

Overall Width (width at mirrors)	84.6 (2,149)

Overall Width (width with folded mirrors)	77.5 (1,968)

Body Width	77.5 (1,968)

Overall Height	71.5 (1,815) — standard suspension

71.5 (1,817) — air suspension

Load Floor Height	32.2 (817) — standard suspension

30.2 (767) — air suspension (with entry/exit enabled)

Sill Step Height	20.2 (513) — standard suspension

18.4 (467) — air suspension (with entry/exit enabled)

Ground Clearance 
(with P265/50R20 tire and 3.6-liter engine)	8.5 (215) — standard suspension

10.9 (276) — air suspension (Pos#2)

Chassis (fuel tank) 	10.3 (261) — standard suspension

12.6 (320) — air suspension (Pos#2)

Front Axle 	9.7 (246) — standard suspension

12.0 (305) — air suspension (Pos#2)

Rear Axle 	10.8 (275) — standard suspension

14.0 (355) — air suspension (Pos#2)

Approach Angle (degrees)	20.6 — standard suspension

30.1 — air suspension (Overland, Pos#2)

28.2 — air suspension (Summit, Pos#2)

Ramp Breakover Angle (degrees)	18.2 — standard suspension

22.6 — air suspension (Pos#2)

### DIMENSIONS AND CAPACITIES (CONTINUED)

Departure Angle (degrees)	21.5 — standard suspension

23.6 — air suspension (Pos#2)

Frontal Area sq. ft. (sq. m)	31.95

Drag Coefficient (Cd)	0.357

Aero CdA	11.4 (Cd x cross-sectional area)

Fuel Tank Capacity, (gallons / liter)	23 (87)

### ACCOMMODATIONS

Seating Capacity (front / rear) 	2/3/2 (second-row bucket seats or optional bench)

SAE Total Interior Passenger Volume, 
(cu. ft. / cu. m)	159.1 (4.5)

Front Row

Headroom	39.8 (1,012)

Legroom	41.3 (1,050)

Shoulder room	59.2 (1,504)

Hip room	57.4 (1,459)

Seat travel	9.8 (250)

EPA front volume index 
(cu. ft. / cu. m)	56.5 (1.6)

Second Row

Headroom	39.9 (1,014)

Legroom	39.4 (1,000)

Knee clearance	4.6 (118)

Shoulder room	58.0 (1,474)

Hip room	56.3 (1,429)

Seat travel	7.1 (180)

EPA rear seat volume index 
(cu. ft. / cu. m)	52.8 (1.5)

Third Row

Headroom	37.3 (947)

Legroom	30.3 (770)

Knee clearance	0.6 (15)

Shoulder room	51.9 (1,317)

Hip room	42.9 (1,090)

EPA rear seat volume index 
(cu. ft. / cu. m)	32.6 (0.92)

### ACCOMMODATIONS (CONTINUED)

Cargo volume behind first-row seats
(cu. ft. / cu. m) 	84.6 (2.39)

Behind second-row seats 
(cu. ft. / cu. m)	46.9 (1.33)

Behind third-row seats 
(cu. ft. / cu. m)	17.2 (0.49)

### WEIGHTS (Estimates)

|  | MODEL | ENGINE | GVWR(a) lbs. (kg) | CURB WEIGHT(b) lbs. (kg) | PAYLOAD(c) lbs. (kg) |
| --- | --- | --- | --- | --- | --- |
| 2WD | Laredo | 3.6-liter | 6,500 (2,948) | 4,524 (2,052) | 1,370 (620) |
|  | Limited | 3.6-liter | 6,500 (2,948) | 4,541 (2,060) | 1,400 (635) |
|  | Overland | 3.6-liter | 6,500 (2,948) | 4,707 (2,135) | 1,230 (560) |
|  | Summit | 3.6-liter | 6,500 (2,948) | 4,752 (2,155) | 1,240 (560) |
| 4WD | Laredo | 3.6-liter | 6,500 (2,948) | 4,658 (2,113) | 1,370 (620) |
|  | Limited | 3.6-liter | 6,500 (2,948) | 4,692 (2,128) | 1,400 (635) |
|  | Overland | 3.6-liter 5.7-liter | 6,700 (3,039) 6,900 (3,130) | 4,997 (2,267) 5,226 (2,370) | 1,270 (575) 1,250 (565) |
|  | Summit | 3.6-liter 5.7-liter | 6,700 (3,039) 6,900 (3,130) | 5,068 (2,299) 5,279 (2,395) | 1,210 (560) 1,190 (540) |

(a) Gross Vehicle Weight Rating

(b) Curb weight includes standard equipment and full quantities of fuel, lubricant and coolant.

(c) Payload is the maximum allowable weight of driver, passengers, cargo and options, rounded to the nearest 10 lbs. (5kg).

### WEIGHT DISTRIBUTION, F/R

|  | MODEL | ENGINE | DISTRIBUTION |
| --- | --- | --- | --- |
| 2WD | Laredo | 3.6-liter | 50/50 |
|  | Limited | 3.6-liter | 50/50 |
|  | Overland | 3.6-liter | 50/50 |
|  | Summit | 3.6-liter | 50/50 |
| 4WD | Laredo | 3.6-liter | 52/48 |
|  | Limited | 3.6-liter | 51/49 |
|  | Overland | 3.6-liter 5.7-liter | 50/50 52/48 |
|  | Summit | 3.6-liter 5.7-liter | 50/50 52/48 |

### Wheels

Availability	Standard — Laredo

Type and material	Painted cast aluminum in Fine Silver

Size (inches)	18 x 8

Availability		Standard — Limited

Type and material	Painted cast aluminum in Fine Silver

Size (inches)	18 x 8

Availability	Optional — Limited

Type and material	Diamond-cut aluminum with High-Gloss Technical Gray pockets

Size (inches)	20 x 8.5

Availability	Standard — Overland

Type and material	Machined cast aluminum with Black Noise pockets

Size (inches)	20 x 8.5

Availability	Optional — Overland (packaged with Off-road Group)

Type and material	Polished aluminum with High-Gloss Black pockets

Size (inches)	18 x 8

Availability	Standard — Summit

Type and material	Painted low-gloss Silver Lithos cast aluminum

Size (inches)	20 x 8.5

Availability 	Standard — Summit Reserve

Type and material	Machined cast aluminum with Mid-Gloss Granite Crystal pockets

Size (inches)	21 x 9

### TIRES

Availability	Standard — Laredo and Limited

Size and type	265/60R18

Mfr. and model	Michelin Primacy XC All-Season or Michelin Primacy LTX All-Season

Revs per kilometer	422

Availability	Standard — Overland; Optional — Limited

Size and type	265/50R20

Mfr. and model	Bridgestone Ecopia H/L 422 Plus Black Sidewall All-Season

Revs per kilometer	425

Availability	Optional — Overland (packaged with Off-road Group)

Size and type	265/60R18

Mfr. and model	Michelin Primacy XC All-Season

Revs per kilometer	422

### TIRES (CONTINUED)

Availability	Standard — Summit

Size and type	265/50R20

Mfr. and model	Pirelli Scorpion Verde Black Sidewall All-Season

Revs per kilometer	419

Availability	Optional — Summit (included with Summit Reserve Package)

Size and type	275/45R21

Mfr. and model	Continental CrossContact

Revs per kilometer	419

Parking Brake Type	Drum-in-hat

Four-wheel Antilock Brakes	Standard

Electronic Stability Control	Standard

All-speed Traction Control	Standard

Brake Assist	Standard

### TRAILER TOWING

|  | Engine | Axle Ratio | Maximum Trailer Weight lbs. (kg) |
| --- | --- | --- | --- |
| 2WD | 3.6-liter V-6 | 3.45 | 6,200 (2,818) |
| 4WD | 3.6-liter V-6 | 3.45 | 6,200 (2,818) |
|  | 5.7-liter V-8 | 3.45 | 7,200 (3,300) |

###