# 2022 Ram 2500 & 3500 Heavy Duty

## towing charts

### RAM 2500 4X2 REGULAR CAB: TRADESMAN 8' BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10000 | 4010 | 5988 | 3400 | 2587 | 5500 | 6000 | 22000 | 15540 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10000 | 4010 | 5988 | 3400 | 2587 | 5500 | 6000 | 24200 | 17740 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 9900 | 3910 | 5988 | 3400 | 2587 | 5500 | 6000 | 22000 | 15540 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 9900 | 3910 | 5988 | 3400 | 2587 | 5500 | 6000 | 24200 | 17740 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 10000 | 3150 | 6850 | 4220 | 2629 | 5500 | 6000 | 27320 | 19990 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 9900 | 3050 | 6850 | 4220 | 2629 | 5500 | 6000 | 27320 | 19990 |

### RAM 2500 4X2 CREW CAB: TRADESMAN W/ RAMBOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10000 | 3540 | 6456 | 3554 | 2902 | 5500 | 6000 | 22000 | 15070 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10000 | 3540 | 6456 | 3554 | 2902 | 5500 | 6000 | 24200 | 17270 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 9900 | 3440 | 6456 | 3554 | 2902 | 5500 | 6000 | 22000 | 15070 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 9900 | 3440 | 6456 | 3554 | 2902 | 5500 | 6000 | 24200 | 17270 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 10000 | 2660 | 7339 | 4367 | 2971 | 5500 | 6000 | 27700 | 19240 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 9900 | 2610 | 7293 | 4262 | 3031 | 5500 | 6000 | 27700 | 18820 |

### RAM 2500 4X2 CREW CAB: TRADESMAN 6'4" BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10000 | 3660 | 6339 | 3560 | 2779 | 5500 | 6000 | 22000 | 15190 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10000 | 3660 | 6339 | 3560 | 2779 | 5500 | 6000 | 24200 | 17390 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 9900 | 3560 | 6343 | 3559 | 2783 | 5500 | 6000 | 22000 | 15190 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 9900 | 3560 | 6343 | 3559 | 2783 | 5500 | 6000 | 24200 | 17390 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 10000 | 2770 | 7229 | 4396 | 2832 | 5500 | 6000 | 27700 | 20000 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 9900 | 2680 | 7223 | 4386 | 2836 | 5500 | 6000 | 27700 | 20000 |

### RAM 2500 4X2 CREW CAB: TRADESMAN 8' BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10000 | 3530 | 6470 | 3684 | 2785 | 5500 | 6000 | 22000 | 15060 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10000 | 3530 | 6470 | 3684 | 2785 | 5500 | 6000 | 24200 | 17260 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 9900 | 3420 | 6477 | 3688 | 2789 | 5500 | 6000 | 22000 | 15050 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 9900 | 3420 | 6477 | 3688 | 2789 | 5500 | 6000 | 24200 | 17250 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 10000 | 2630 | 7368 | 4528 | 2840 | 5500 | 6000 | 27840 | 20000 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 9900 | 2520 | 7377 | 4534 | 2843 | 5500 | 6000 | 27840 | 19900 |

### RAM 2500 4X4 MEGA CAB: BIG HORN / LONE STAR W/ RAMBOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10000 | 2810 | 7191 | 4073 | 3117 | 5500 | 6390 | 22000 | 14340 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10000 | 2810 | 7191 | 4073 | 3117 | 5500 | 6390 | 24200 | 16530 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 9900 | 2710 | 7191 | 4073 | 3117 | 5500 | 6390 | 22000 | 14340 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 9900 | 2710 | 7191 | 4073 | 3117 | 5500 | 6390 | 24200 | 16530 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 10000 | 1920 | 8076 | 4897 | 3179 | 6000 | 6040 | 24240 | 14480 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 9900 | 1820 | 8076 | 4897 | 3179 | 6000 | 6040 | 24240 | 13480 |

### RAM 2500 4X4 MEGA CAB: BIG HORN / LONE STAR 6'4" BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10000 | 2940 | 7061 | 4061 | 3000 | 5500 | 6390 | 22000 | 14470 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10000 | 2940 | 7061 | 4061 | 3000 | 5500 | 6390 | 24200 | 16600 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 9900 | 2840 | 7061 | 4061 | 3000 | 5500 | 6390 | 22000 | 14470 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 9900 | 2840 | 7061 | 4061 | 3000 | 5500 | 6390 | 24200 | 16600 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 10000 | 2040 | 7963 | 4907 | 3055 | 6000 | 6040 | 24240 | 15620 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 9900 | 1940 | 7963 | 4907 | 3055 | 6000 | 6040 | 24240 | 14620 |

### RAM 2500 4X4 REGULAR CAB: TRADESMAN 8' BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10000 | 3690 | 6306 | 3686 | 2620 | 5500 | 6000 | 22000 | 15220 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10000 | 3690 | 6306 | 3686 | 2620 | 5500 | 6000 | 24200 | 17420 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 9900 | 3580 | 6318 | 3693 | 2624 | 5500 | 6000 | 22000 | 15210 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 9900 | 3580 | 6318 | 3693 | 2624 | 5500 | 6000 | 24200 | 17410 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 10000 | 2810 | 7194 | 4520 | 2673 | 5750 | 6000 | 27650 | 19980 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 9900 | 2710 | 7190 | 4523 | 2666 | 5750 | 6000 | 27650 | 19980 |

### RAM 2500 4X4 CREW CAB: POWER WAGON W/ RAMBOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 8565 | 1520 | 7045 | 4087 | 2958 | 4750 | 6200 | 18000 | 10440 |

### RAM 2500 4X4 CREW CAB: POWER WAGON 6'4" BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 8565 | 1630 | 6939 | 4086 | 2852 | 4750 | 6200 | 18000 | 10590 |

### RAM 2500 4X4 CREW CAB: TRADESMAN 8' BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10000 | 3170 | 6825 | 3995 | 2830 | 5500 | 6000 | 22000 | 14700 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10000 | 3170 | 6825 | 3995 | 2830 | 5500 | 6000 | 24200 | 16900 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 9900 | 3080 | 6815 | 3992 | 2823 | 5500 | 6000 | 22200 | 14710 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 9900 | 3080 | 6815 | 3992 | 2823 | 5500 | 6000 | 24200 | 16910 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 10000 | 2300 | 7698 | 4824 | 2874 | 5750 | 6000 | 26400 | 18230 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 9900 | 2200 | 7703 | 4827 | 2876 | 5750 | 6000 | 26400 | 17210 |

# 2022 Ram 3500 Heavy Duty

## TOWING CHART

### RAM 3500 4X2 REGULAR CAB: TRADESMAN 8' BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 14000 | 7680 | 6319 | 3396 | 2922 | 5500 | 9750 | 22600 | 15810 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 14000 | 7680 | 6319 | 3396 | 2922 | 5500 | 9750 | 25000 | 18210 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10700 | 4640 | 6056 | 3414 | 2642 | 5500 | 7000 | 22000 | 15470 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10700 | 4640 | 6056 | 3414 | 2642 | 5500 | 7000 | 24200 | 17670 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 11500 | 4610 | 6893 | 4227 | 2666 | 5500 | 7000 | 28300 | 20940 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 14000 | 6830 | 7170 | 4225 | 2944 | 5500 | 9750 | 28300 | 20660 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 4.10 | 14000 | 6830 | 7170 | 4225 | 2944 | 5500 | 9750 | 30300 | 22660 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 13500 | 6330 | 7171 | 4225 | 2945 | 5500 | 9750 | 28300 | 20660 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 4.10 | 13500 | 6330 | 7171 | 4225 | 2945 | 5500 | 9750 | 30300 | 22660 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 11500 | 4490 | 7012 | 4290 | 2722 | 5500 | 7000 | 34630 | 26780 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 14000 | 6560 | 7440 | 4329 | 3110 | 5500 | 9750 | 41600 | 33690 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 4.10 | 14000 | 6560 | 7440 | 4329 | 3110 | 5500 | 9750 | 45000 | 37090 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 13500 | 6060 | 7441 | 4330 | 3111 | 5500 | 9750 | 41600 | 33690 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 4.10 | 13500 | 6060 | 7441 | 4330 | 3111 | 5500 | 9750 | 45000 | 35250 |

### RAM 3500 4X2 CREW CAB: TRADESMAN W/ RAMBOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10700 | 4150 | 6545 | 3583 | 2961 | 5500 | 7000 | 22000 | 14980 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10700 | 4150 | 6545 | 3583 | 2961 | 5500 | 7000 | 24200 | 17180 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 11500 | 4030 | 7465 | 4454 | 3011 | 5500 | 7000 | 28300 | 20360 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 11500 | 3940 | 7560 | 4492 | 3067 | 5500 | 7000 | 32240 | 23130 |

### RAM 3500 4X2 CREW CAB: TRADESMAN 6'4"BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 10700 | 4280 | 6424 | 3617 | 2807 | 5500 | 7000 | 22200 | 15110 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 10700 | 4280 | 6424 | 3617 | 2807 | 5500 | 7000 | 24200 | 17310 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 11500 | 4180 | 7320 | 4432 | 2888 | 5500 | 7000 | 28300 | 20510 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 11500 | 4080 | 7415 | 4470 | 2944 | 5500 | 7000 | 32240 | 24100 |

### RAM 3500 4X2 CREW CAB: TRADESMAN 8' BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 11100 | 4510 | 6593 | 3742 | 2850 | 5500 | 7000 | 22000 | 14940 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 11100 | 4510 | 6593 | 3742 | 2850 | 5500 | 7000 | 24200 | 17140 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 14000 | 7050 | 6949 | 3763 | 3186 | 5500 | 9750 | 22600 | 15180 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 14000 | 7050 | 6949 | 3763 | 3186 | 5500 | 9750 | 25000 | 17580 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 12000 | 4540 | 7463 | 4569 | 2893 | 5500 | 7000 | 28300 | 20370 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 14000 | 6180 | 7821 | 4568 | 3252 | 5500 | 9750 | 28300 | 20010 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 4.10 | 14000 | 6180 | 7821 | 4568 | 3252 | 5500 | 9750 | 30300 | 22010 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 13500 | 5680 | 7822 | 4569 | 3253 | 5500 | 9750 | 28300 | 20010 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 4.10 | 13500 | 5680 | 7822 | 4569 | 3253 | 5500 | 9750 | 30300 | 22010 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 12000 | 4440 | 7559 | 4616 | 2943 | 5500 | 7000 | 33670 | 25560 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 14000 | 5930 | 8066 | 4693 | 3372 | 5500 | 9750 | 41600 | 33060 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 4.10 | 14000 | 5930 | 8066 | 4693 | 3372 | 5500 | 9750 | 43000 | 34460 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 13500 | 5430 | 8067 | 4693 | 3373 | 5500 | 9750 | 41600 | 33060 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 4.10 | 13500 | 5430 | 8067 | 4693 | 3373 | 5500 | 9750 | 43000 | 33080 |

### RAM 3500 4X4 MEGA CAB: BIG HORN / LONE STAR W/ RAMBOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 11400 | 4280 | 7123 | 4009 | 3113 | 5500 | 7000 | 22000 | 14110 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 11400 | 4280 | 7123 | 4009 | 3113 | 5500 | 7000 | 24200 | 16610 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 12300 | 4110 | 8190 | 4953 | 3237 | 6000 | 7000 | 28300 | 19640 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 12300 | 3970 | 8326 | 5049 | 3276 | 6000 | 7000 | 32710 | 23300 |

### RAM 3500 4X4 MEGA CAB: BIG HORN / LONE STAR 6'4" BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 14000 | 6630 | 7373 | 4033 | 3340 | 5500 | 9750 | 22600 | 14760 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 14000 | 6630 | 7373 | 4033 | 3340 | 5500 | 9750 | 25000 | 17160 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 11400 | 4340 | 7063 | 4049 | 3013 | 5500 | 7000 | 22000 | 14470 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 11400 | 4340 | 7063 | 4049 | 3013 | 5500 | 7000 | 24200 | 16670 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 12300 | 4290 | 8006 | 4904 | 3101 | 6000 | 7000 | 28300 | 19820 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 14000 | 5660 | 8335 | 4876 | 3458 | 6000 | 9750 | 28300 | 19490 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 4.10 | 14000 | 5660 | 8335 | 4876 | 3458 | 6000 | 9750 | 30300 | 21490 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 13500 | 5160 | 8336 | 4876 | 3459 | 6000 | 9750 | 28300 | 19490 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 4.10 | 13500 | 5160 | 8336 | 4876 | 3459 | 6000 | 9750 | 30300 | 21490 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 12300 | 4130 | 8165 | 5013 | 3151 | 6000 | 7000 | 32710 | 24070 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 14000 | 5440 | 8557 | 5021 | 3536 | 6000 | 9750 | 41600 | 32570 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 4.10 | 14000 | 5440 | 8557 | 5021 | 3536 | 6000 | 9750 | 41955 | 32930 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 13500 | 4940 | 8559 | 5021 | 3537 | 6000 | 9750 | 41600 | 29810 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 4.10 | 13500 | 4940 | 8559 | 5021 | 3537 | 6000 | 9750 | 41955 | 29810 |

### RAM 3500 4X4 REGULAR CAB: TRADESMAN 8' BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 11000 | 4590 | 6407 | 3728 | 2679 | 5500 | 7000 | 22000 | 15120 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 11000 | 4590 | 6407 | 3728 | 2679 | 5500 | 7000 | 24200 | 17320 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 14000 | 7230 | 6769 | 3778 | 2991 | 5500 | 9750 | 22600 | 15360 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 14000 | 7230 | 6769 | 3778 | 2991 | 5500 | 9750 | 25000 | 17760 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 14000 | 6400 | 7604 | 4540 | 3064 | 6000 | 9750 | 28300 | 20230 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 4.10 | 14000 | 6400 | 7604 | 4540 | 3064 | 6000 | 9750 | 30300 | 22230 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 11800 | 4510 | 7286 | 4552 | 2733 | 6000 | 7000 | 28300 | 20540 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 13500 | 5890 | 7606 | 4540 | 3065 | 6000 | 9750 | 28300 | 20220 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 4.10 | 13500 | 5890 | 7606 | 4540 | 3065 | 6000 | 9750 | 30300 | 22220 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 14000 | 6120 | 7880 | 4700 | 3180 | 6000 | 9750 | 41600 | 33250 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 4.10 | 14000 | 6120 | 7880 | 4700 | 3180 | 6000 | 9750 | 43000 | 34650 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 11800 | 4370 | 7434 | 4637 | 2797 | 6000 | 7000 | 34540 | 25970 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 13500 | 5620 | 7882 | 4700 | 3181 | 6000 | 9750 | 41600 | 33250 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 4.10 | 13500 | 5620 | 7882 | 4700 | 3181 | 6000 | 9750 | 43000 | 34320 |

### RAM 3500 4X4 CREW CAB: TRADESMAN W/ RAMBOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 11000 | 4140 | 6862 | 3871 | 2991 | 5500 | 7000 | 22000 | 14670 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 11000 | 4140 | 6862 | 3871 | 2991 | 5500 | 7000 | 24200 | 16870 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 11800 | 4050 | 7750 | 4691 | 3059 | 6000 | 7000 | 28300 | 20080 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 11800 | 3870 | 7925 | 4803 | 3122 | 6000 | 7000 | 32580 | 23000 |

### RAM 3500 4X4 CREW CAB: TRADESMAN 6'4" BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 11000 | 4250 | 6745 | 3886 | 2859 | 5500 | 7000 | 22000 | 14780 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 11000 | 4250 | 6745 | 3886 | 2859 | 5500 | 7000 | 24200 | 16980 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 11800 | 4190 | 7605 | 4693 | 2912 | 6000 | 7000 | 28300 | 20220 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 11800 | 4010 | 7789 | 4806 | 2982 | 6000 | 7000 | 32580 | 23600 |

### RAM 3500 4X4 CREW CAB: TRADESMAN 8' BOX

| Engine | Transmissions | Axle Ratio | GVWR | Payload | Base Weight Total | Base Weight Front | Base Weight Rear | GAWR Front | GAWR Rear | GCWR | Max. Trailer Weight Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 14000 | 6700 | 7295 | 4068 | 3226 | 5500 | 9750 | 22600 | 14830 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 14000 | 6700 | 7295 | 4068 | 3226 | 5500 | 9750 | 25000 | 17230 |
| 6.4L V-8 HEMI | A8 8HP75 | 3.73 | 11400 | 4450 | 6952 | 4053 | 2899 | 5500 | 7000 | 22000 | 14580 |
| 6.4L V-8 HEMI | A8 8HP75 | 4.10 | 11400 | 4450 | 6952 | 4053 | 2899 | 5500 | 7000 | 24200 | 16780 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 12300 | 4490 | 7809 | 4856 | 2953 | 6000 | 7000 | 28300 | 20020 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 14000 | 5850 | 8149 | 4862 | 3287 | 6000 | 9750 | 28300 | 19680 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 4.10 | 14000 | 5850 | 8149 | 4862 | 3287 | 6000 | 9750 | 30300 | 21680 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 3.73 | 13500 | 5350 | 8150 | 4862 | 3288 | 6000 | 9750 | 28300 | 19680 |
| 6.7L I-6 TURBO DIESEL | A6 68RFE | 4.10 | 13500 | 5350 | 8150 | 4862 | 3288 | 6000 | 9750 | 30300 | 21680 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 12300 | 4330 | 7974 | 4964 | 3010 | 6000 | 7000 | 33610 | 25120 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 14000 | 5580 | 8420 | 4996 | 3423 | 6000 | 9750 | 41600 | 32710 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 4.10 | 14000 | 5580 | 8420 | 4996 | 3423 | 6000 | 9750 | 43000 | 34070 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 3.73 | 13500 | 5080 | 8421 | 4996 | 3424 | 6000 | 9750 | 41600 | 30720 |
| 6.7L I-6 HO TURBO DIESEL | A6 AS69RC | 4.10 | 13500 | 5080 | 8421 | 4996 | 3424 | 6000 | 9750 | 43000 | 30720 |

NOTES

- Payload and Max Trailer Weights are ESTIMATED values.

- All values are shown in pounds unless otherwise stated.

- Payload and Max Trailer Weight values are rounded to the nearest 10 lb.

- Payload = GVWR – Base Weight.

- Payload and Trailer Weight Rating are mutually exclusive.

- GAWRs, GVWRs and GCWRs should never be exceeded.

- Trailer Weight Rating and Tow Vehicle Trailering Weight are calculated as specified in SAE J2807.

- Passenger Weight = 300 lb.

- Options Weight = 100 lb.

- Trailering Equipment Weight: 75 lb. for Conventional Hitch, 70 lb. for Gooseneck 
and 250 lb. for 5th Wheel.

- Tongue weight: 10 percent of the gross trailer weight for Conventional Hitch,
15 percent of the gross trailer weight for a 5th Wheel or Gooseneck hitch.

- Payload and GAWR should never be exceeded and must account for all of the above weights, including the appropriate trailering equipment and tongue weight.

- Box Off Body Completion Weight = 80 lb. per foot from end of cab to end of frame.

- A fifth-wheel hitch is required for trailers over 23,000 lbs.

- A Gooseneck hitch is required for trailers over 30,000 lbs.

- Maximum Capability

WEIGHT TERM DEFINITIONS

33% option weight

Combined weight of all production installed options that have a projected sales volume of more than 33% and comply with EPA rules.

Axle Ratio

The ratio between the driveshaft rpm and axle shaft rpm. The axle ratio is critical in the operation of the transmission/driveshaft/axle system that transmits engine torque to the driving wheels.

Base Weight

The weight of a standard equipped sales unit (vehicle line, body style, GVWR, engine, transmission & electrification) with full quantities of fuel, lubricant & coolant.

Gross Axle Weight Rating (GAWR)

The maximum weight to be carried by a single axle (front or rear).

Gross Combined Weight Rating (GCWR)

The maximum allowable weight of the towing vehicle and the loaded trailer – including all cargo and passengers – that the vehicle can handle without risking damage.

Gross Vehicle Weight Rating (GVWR)

The maximum allowable weight of the fully loaded vehicle.

Payload

The combined maximum allowable weight of options, cargo and passengers that the truck is designed to carry.

Tongue Weight

The weight a conventional trailer puts on the back of a tow vehicle. 10 percent of the trailer weight recommended for a conventional hitch trailer and 15 percent recommended for a gooseneck or fifth wheel trailer.

Trailer Weight Rating (TWR)

The maximum allowable weight of a trailer plus cargo that a vehicle configuration can handle.

###