# 2022 Wagoneer / Grand Wagoneer

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Vehicle Type	Four-door sport-utility vehicle

Assembly Plant	Warren Truck Assembly Plant, Warren, Michigan

EPA Vehicle Class	Multipurpose vehicle

Introduction Date	March 2021 as a 2022 model

### BODY AND CHASSIS

Layout	Front engine, rear- or four-wheel drive

Construction	Steel frame

### ENGINE: 5.7-liter HEMI® V-8 WITH ETORQUE HYBRID ASSIST

Availability	Standard — Wagoneer

Type and Description	90-degree V-8 with variable cam timing

Hybrid Battery	48-volt, 12-cell lithium-ion, nickel manganese cobalt (NMC) 
graphite chemistry, 390Wh

Belt-starter Generator	12kW power, 130 lb.-ft. launch torque

Displacement	345 cu. in. (5,654 cu. cm)

Bore x Stroke	3.92 x 3.58 (99.5 x 90.9)

Valve System	Variable-cam timing, pushrod-operated overhead valves, 
16 valves, hydraulic lifters with roller followers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, 
aluminum-alloy heads with hemispherical combustion chambers

Compression Ratio	10.5:1

Power	392 hp (292 kW) @ 5,600 rpm

Torque	404 lb.-ft. (548 N•m) @ 3,950 rpm

Max. Engine Speed	5,800 rpm

Fuel Requirement	Unleaded mid-grade, 89 octane (R+M)/2 — recommended

Unleaded regular, 87 octane (R+M)/2 — acceptable

### ENGINE: 5.7-liter HEMI® V-8 WITH ETORQUE HYBRID ASSIST (continued)

Oil Capacity	7.0 quarts (6.6 liters)

Coolant Capacity	17.2 liter (15.6 quarts)

Factory Oil Fill 	Pennzoil Gold 0W-20

Emission Controls	Three-way catalytic converters, heated oxygen sensors 
and internal engine features

EPA Fuel Economy mpg (city/hwy)	TBD

Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

### ENGINE: 6.4-LITER V-8

Availability	Standard — Grand Wagoneer

Type and Description	90-degree V-type, liquid-cooled

Displacement, cu. in. (cu. cm.)	392 (6,417)

Bore x Stroke	4.09 x 3.72 (103.9 x 94.5)

Valve System	Pushrod-operated overhead valves, 16 valves with sodium-filled exhaust valves and hollow stem intake valves, 16 conventional hydraulic lifters, all with roller tips

Fuel Injection	Sequential, multi-port, electronic, returnless; 
features Fuel Saver mode

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, 
aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	10.9:1

Power (SAE J2723)	471 hp (351 kW) @ 6,000 rpm

Torque (SAE J2723)	455 lb.-ft. (617 N•m) @ 4,400 rpm

Max. Engine Speed	6,400 rpm (electronically limited)

Fuel Requirement	Premium 91 octane (R+M)/2 — recommended

Oil Capacity, qt. (liter) 	7 (6.6)

Coolant Capacity, qt. (liter)	18.1 liter (16.4 quarts)

Factory Oil Fill 	Pennzoil Ultra Platinum 0W-40

Emission Controls 	Dual close-coupled three-way catalytic converters, 
quad-heated oxygen sensors and internal engine features(a)

Exhaust Type 	Dual exhaust with dual 4-in. oval black chrome tips

EPA Fuel Economy mpg (city/hwy/combined)	TBD

Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

(a)	Meets Federal Tier 2, Bin 5 emissions requirements; marketed in California as an LEV II (Low Emission Vehicle) 
under cleanest vehicle rules.

### TRANSMISSION: TORQUEFLITE 8HP75 AUTOMATIC, EIGHT-SPEED OVERDRIVE

Availability	Standard — 5.7-liter V-8 with eTorque mild-hybrid assist

Description	5 clutches and 4 planetary gear set architecture. 7.03 overall ratio spread. Start/stop and electronic range select capability. Electronically modulated torque converter clutch with torsional turbine damper

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.11

4th	1.67

5th	1.29

6th	1.00

7th	0.84

8th	0.67

Reverse 	3.30

### TRANSMISSION: TORQUEFLITE 8HP75 AUTOMATIC, EIGHT-SPEED OVERDRIVE

Availability	Standard — 6.4-liter V-8 models

Description	5 clutches and 4 planetary gear set architecture. 7.03 overall ratio spread. Electronically modulated torque converter clutch with torsional turbine damper. Electronic range select capability.

Gear Ratios

1st	4.71

2nd 	3.14

3rd	2.10

4th 	1.67

5th 	1.29

6th	1.00

7th	0.84

8th 	0.67

Reverse 	3.30

### TRANSFER CASE: 1-SPEED

Availability 	Optional — 5.7-liter V-8 engine

Type 	Single-speed

Operating Mode 	Full-time active 4x4

Torque Split, Front/Rear	Variable (60/40) — Auto, Tow, Snow, Mud, Sand, Rock, Sport

Low Range Ratio 	None

### TRANSFER CASE: 2-SPEED

Availability 	Optional — 5.7-liter V-8 engine

Optional — 6.4-liter V-8 engine

Type 	Two-speed electronically shifted

Operating Modes 	4x4 Low Active, Neutral, full-time active 4x4

Low Range Ratio	2.64

Torque Split, Front/Rear 	Variable (50/50 or 100 F/R) — Auto, Tow, Snow, Mud, 
Sand, Rock, Sport

### AXLES

Front

Differential Type	Open

Availability 	Standard on 4x4 models

Ring Gear Diameter	8.5 (215)

Axle Ratios	3.21:1 — Standard with 5.7-liter V-8

3.92:1 — Optional with 5.7-liter V-8, standard with 6.4-liter V-8

Rear

Differential Type 	Limited Slip or Electronic Limited Slip (ELSD)

Availability 	Limited Slip Differential — Standard with 5.7-liter V-8

Electronic Limited Slip Differential (ELSD) — Optional with 5.7-liter V-8, Standard with 6.4-liter V-8

Ring Gear Diameter (in/mm) 	10.2 (260)

Axle Ratios 	3.21:1 — Standard with 5.7-liter V-8

3.92:1 — Optional with 5.7-liter V-8, standard with 6.4-liter V-8

### ELECTRICAL SYSTEM

Alternator	240-amp

Battery	Maintenance-free

H7 730 CCA

### SUSPENSION

Front	Short- and long-arm independent with hybrid steel composite upper control arm, aluminum lower control arm, aluminum knuckle, coil springs with monotube shocks or Quadra-lift air suspension with 
semi-active damping, solid or hollow stabilizer bar

Rear	Five-link independent rear suspension, coil springs with monotube load leveling shocks or Quadra-lift air suspension with semi-active damping, solid or hollow stabilizer bar, cast aluminum links (tension, compression, camber, toe), high strength steel spring link

### STEERING

Type	Electric rack-and-pinion steering

Overall Ratio	16.7:1

Turning Diameter (curb-to-curb) (ft./m)	38.0 (11.6)

Steering Turns (lock-to-lock)	3.2

### BRAKES

Type	Electronic power brakes (eBooster), anti-lock brake system (ABS)

Availability	Standard

Front

Rotor size and type Inches (mm)	14.88 x 1.18 (378 x 30) vented disc

Caliper size and type Inches (mm)	2.24 (57) two-piston pin-slider caliper

Swept area, (sq. in. / sq. cm)	100.9 (651.5)

Rear

Rotor size and type Inches (mm)	14.76 x 0.87 (375 x 22) solid disc

Caliper size and type Inches (mm)	2.24 (57) single-piston pin-slider caliper with EPB

Swept area, (sq. in. / sq. cm)	79.2 (511.6)

### DIMENSIONS AND CAPACITIES

Wheelbase	123.0 (3,124)

Track, Front	68.5 (1,740)

Track, Rear	68.3 (1,734)

Overall Length	214.7 (5,453)

Overall Width (width at mirrors)	94.0 (2,388)

Body Width	83.6 (2,124)

### DIMENSIONS AND CAPACITIES (continued)

Overall Height (at roof rail / at antenna)	75.6 (1,921) / 77.3 (1964)

Load Floor Height	34.0 (864) — standard suspension

31.9 (810) — air suspension

Sill Step Height	22.8 (579) — standard suspension

21.1 (537) — air suspension

Ground Clearance (with 275/55R20 
tire and 5.7-liter engine)	8.3 (211) — standard suspension

10.0 (254) — air suspension (Pos#2)

Chassis (fuel tank) 	9.7 (246) — standard suspension

10.9 (277) — air suspension (Pos#2)

Front Axle 	9.6 (243) — standard suspension

11.6 (294) — air suspension (Pos#2)

Rear Axle 	9.0 (229) — standard suspension

11.0 (279) — air suspension (Pos#2)

Approach Angle (degrees)	21.5 — standard suspension

25.0 — air suspension (Pos#2)

25.0 (Off-road II with air dam on)

Ramp Breakover Angle (degrees)	18.5 — standard suspension

22.0 — air suspension (Pos#2)

Departure Angle (degrees)	21.1 — standard suspension

24.0 — air suspension (Pos#2)

Frontal Area sq. ft. (sq. m)	37.2 (3.46)

Drag Coefficient (Cd)	0.352 — Wagoneer

Aero CdA	13.1 (Cd x cross-sectional area) — Wagoneer

Fuel Tank Capacity, (gallons / liter)	26.5 (100.3)

### ACCOMMODATIONS

Seating Capacity	Wagoneer

Standard — 2/3/3 (including 2nd row bench)

Optional — 2/2/3 (including captain’s chairs)

Grand Wagoneer

Standard — 2/2/3 (including captain’s chairs)

Optional — 2/3/3 (including 2nd row bench)

### ACCOMMODATIONS (continued)

SAE Total Interior Passenger Volume, 
(cu. ft. / cu. m)	179.2 (5.1)

Front

Headroom	41.3 (1049)

Legroom	40.9 (1040)

Shoulder room	66.1 (1680)

Hip room	63.4 (1611)

Seat travel	8.1 (207)

EPA front volume index (cu. ft. / cu. m)	64.7 (1.8)

Cargo volume behind first-row seats 
(cu. ft. / cu. m)	116.7 (3.3) Wagoneer / 94.2 (2.7) Grand Wagoneer

2nd Row

Headroom	40.0 (1016)

Legroom	42.7 (1085)

Knee clearance	6.7 (170) Wagoneer / 5.6 (143) Grand Wagoneer

Shoulder room	65.2 (1657)

Hip room	63.0 (1599)

EPA 2nd row seat volume index 
(cu. ft. / cu. m)	64.5 (1.8)

Cargo volume behind second-row seats 
(cu. ft. / cu. m)	70.8 (2.0) Wagoneer / 70.9 (2.0) Grand Wagoneer

3rd Row

Headroom	39.0 (991)

Legroom	36.6 (930)

Knee clearance	1.9 / 47

Shoulder room	64.4 (1635)

Hip room	51.6 (1310)

EPA 3rd row seat volume index 
(cu. ft. / cu. m)	50.0 (1.4)

Cargo volume behind third-row seats 
(cu. ft. / cu. m)	27.4 (0.8)

Total passenger plus cargo volume 
(cu. ft. / cu. m)	206.6 / 5.9

### WEIGHTS (Estimates)

| DRIVE SYSTEM | MODEL | ENGINE | GVWR lbs. (kg) | CURB WEIGHT(b) | PAYLOAD(c) lbs. (kg) | WEIGHT DISTRIBUTION, F/R |
| --- | --- | --- | --- | --- | --- | --- |
| 2WD | Wagoneer Series II | 5.7-liter | 7,400 (3357) | 5,960 (2703) | 1,510 (685) | 51/49 |
|  | Wagoneer Series III | 5.7-liter | 7,500 (3402) | 6,000 (2722) | 1,580 (717) | 51/49 |
| 4WD | Wagoneer Series II | 5.7-liter | 7,650 (3470) | 6,190 (2808) | 1,550 (703) | 52/48 |
|  | Wagoneer Series III | 5.7-liter | 7,700 (3493) | 6,230 (2826) | 1,560 (708) | 52/48 |
| 4WD | Grand Wagoneer Series I | 6.4-liter | 7,700 (3943) | 6,340 (2876) | 1,450 (658) | 51/49 |
|  | Grand Wagoneer Series II | 6.4-liter | 7,700 (3493) | 6,400 (2903) | 1,380 (626) | 51/49 |
|  | Grand Wagoneer Series III | 6.4-liter | 7,700 (3493) | 6,420 (2912) | 1,360 (617) | 51/49 |

(b)	Curb weight includes standard equipment and full quantities of fuel, lubricant and coolant.

(c)	Payload is the maximum allowable weight of driver, passengers, cargo, and options, rounded to the nearest 10 lbs. (5kg).

### Wheels

Availability 		Standard on Wagoneer Series II

Type and material	Painted cast aluminum, six spoke

Size (in.)	20 x 9

Availability	Optional on Wagoneer Series III

Type and material	Machined cast aluminum with Black Noise painted pockets

Size (in.)	20 x 9

Availability	Standard on Grand Wagoneer Series I

Type and material	Machined cast aluminum with Baltic Gray painted pockets

Size (in.)	20 x 9

Availability 	Standard on Grand Wagoneer Series II, optional on 
Grand Wagoneer Series I

Type and material	Polished aluminum with Luster Gray painted pockets

Size (in.)	22 x 9

Availability	Standard on Grand Wagoneer Series III

Type and material	Polished aluminum with Lights Out painted pockets

Size (in.)	22 x 9

Availability	Optional on Wagoneer Series II, optional on Wagoneer Series III

Type and material	Polished aluminum with Black Noise painted pockets

Size (in.)	22 x 9

### Wheels (continued)

Availability	Optional on Grand Wagoneer Series II

Type and material	Painted cast aluminum Satin Carbon with chrome inserts

Size (in.)	22 x 9

Availability		Optional on Grand Wagoneer Series III

Type and material	Machined cast aluminum with Black Noise painted pockets and brushed nano steel inserts

Size (in.)	22 x 9

Availability	Optional with Off-road Package

Type and material	Machined cast aluminum with Black Noise painted pockets

Size (in.)	18 x 8

Availability	Optional with Off-road Package

Type and material	Machined cast aluminum with Satin Granite Crystal painted pockets

Size (in.)	20 x 9

Availability	Standard Onyx

Type and material	Painted cast aluminum with Gloss Black

Size (in.)	22 x 9

Availability	Standard Obsidian

Type and material	Tinted machined cast aluminum with Black Noise painted pockets 
and Gloss Black inserts

Size (in.)	22 x 9

### TIRES

Availability	Standard – Wagoneer Series II, Wagoneer Series III

Size and type	275/55R20

Mfr. And model	Nexen Roadian HTX RH5 All Season

Revs per mile	656

Availability 	Standard – Grand Wagoneer Series I; Packaged – HD Trailer Tow, 
Premium Group I

Size and type	275/55R20

Mfr. And model	Bridgestone Dueler H/L Alenza All Season T Rated for NA

Revs per mile	655

Availability	Standard – Grand Wagoneer Series II, Grand Wagoneer Series III; 
Packaged – Premium Group I

Size and type	285/45R22XL

Mfr. and model	Goodyear Eagle Touring All Season/Perf

Revs per mile	639

### TIRES (CONTINUED)

Availability 	Packaged – Advanced All Terrain Group

Size and type 	275/65R18

Mfr. and model 	Firestone Destination AT2 All Terrain

Revs per mile 	651

Availability	Packaged – Advance All Terrain Group

Size and type	275/55R20

Mfr. and model	Falken Wildpeak A/T All Terrain

Revs per mile	660

Parking Brake Type	Electric power steering (standard)

Four-wheel Anti-lock Brakes	Standard

Electronic Stability Control	Standard

All-speed Traction Control	Standard

Brake Assist	Standard

### TRAILER TOWING

| Drive System | Engine | Axle Ratio | Maximum Trailer Weight lbs. (kg) |
| --- | --- | --- | --- |
| 2WD | 5.7-liter V-8 | 3.21 | 10,000 lbs. (4536kg) |
| 4WD | 5.7-liter V-8 | 3.21 | 10,000 lbs. (4536kg) |
| 4WD | 6.4-liter V-8 | 3.92 | 9,850 lbs. (4472kg) |

###